# Chapter: 5G RAN Dataset Generation and Offloading Methodology

## 1. Overview and Objectives

This section details the complete end-to-end data collection, simulation, metric parsing, dataset augmentation, cost modeling, and labeling methodology for a 5G Radio Access Network (RAN) environment. The generated dataset facilitates research in smart computational offloading (Mobile vs. Edge vs. Cloud) using supervised machine learning and Explainable AI (XAI).

Dataset Link: Access the complete augmented dataset on Google Drive: 5G Smart Offloading Dataset (Google Drive) [cite: 1]

## 2. Real 5G srsRAN Data Collection Procedure

The physical measurements were generated by deploying a simulated 5G RAN environment via srsRAN_Project on an Ubuntu system [cite: 1]. The step-by-step workflow followed during empirical data collection is detailed below [cite: 1]:

### Step 1 – Launch Environment

Launch Ubuntu terminal and verify current working directory [cite: 1]:

pwd

Expected Output: /root [cite: 1]

### Step 2 – Activate Python Virtual Environment

Navigate to the project directory and activate the virtual environment [cite: 1]:

cd ~/5g_ai_dataset
source venv/bin/activate

Expected Output: (venv) root@SURYAKUMAR:~/5g_ai_dataset# [cite: 1]

### Step 3 – Open srsRAN Project

Change directory to the srsRAN project codebase [cite: 1]:

cd ~/srsRAN_Project
pwd

Expected Output: /root/srsRAN_Project [cite: 1]

### Step 4 – Configure Scenario Parameters

Edit test mode configurations to adjust network conditions (e.g., CQI and number of UEs) [cite: 1]:

nano configs/testmode.yml

Sample Configuration [cite: 1]:

test_mode:
  test_ue:
    rnti: 0x44
    ri: 1
    cqi: 15
    nof_ues: 5
    pusch_active: true
    pdsch_active: true

Modify parameters such as cqi (e.g., set to 10) and nof_ues (e.g., set to 5), then save with CTRL + O, press Enter, and exit with CTRL + X [cite: 1].

### Step 5 – Start srsRAN gNB

Execute the gNodeB (gNB) application in test mode [cite: 1]:

./build/apps/gnb/gnb -c configs/testmode.yml -c configs/debug.yml

### Step 6 – Generate Traffic and Collect Live Statistics

After gNB initialization, interact with the console by pressing t, entering ue, and pressing Enter to display live UE statistics and generate network traffic for 30–60 seconds per test run [cite: 1].

### Step 7 – Stop Data Collection

Press q to terminate the run [cite: 1]. Logfiles and MAC PCAP files are generated and stored at [cite: 1]:

- /tmp/gnb.log [cite: 1]

- /tmp/gnb_mac.pcap [cite: 1]

### Step 8 – Parse Raw Log Measurements

Extract radio metrics into structured CSV format [cite: 1]:

cd ~/5g_ai_dataset
python3 parser_v2.py

Generates: dataset_v2.csv [cite: 1]

### Step 9 – Store Scenario-Specific Output

Save parsed metrics into organized scenario folders [cite: 1]:

mkdir -p scenarios
cp dataset_v2.csv scenarios/cqi10_ue5.csv

### Step 10 – Multi-Scenario Execution Matrix

The data collection workflow was executed across 24 distinct network scenarios combining various Channel Quality Indicator (CQI) levels and User Equipment (UE) loads [cite: 1]:

| CQI Values | UE Load Configurations | Scenario Count |
| --- | --- | --- |
| 15, 12, 10, 8, 5, 3 | 1, 5, 10, 20 UEs per CQI level | 24 Scenarios |

### Step 11 – Merge Scenario Datasets

Consolidate all individual scenario files into a unified dataset [cite: 1]:

cd ~/5g_ai_dataset
rm -f final_dataset_24.csv
head -1 scenarios/cqi15_ue1.csv > final_dataset_24.csv
for f in scenarios/cqi*_ue*.csv
do
    tail -n +2 "$f" >> final_dataset_24.csv
done
wc -l final_dataset_24.csv

### Step 12 – Verify Measurement Distribution

Check the distribution across CQI values [cite: 1]:

cut -d',' -f1 final_dataset_24.csv | sort | uniq -c

### Step 13 & 14 – Base Labeling & Machine Learning Validation

Initial labeling and ML validation script executions [cite: 1]:

python add_labels_v3.py
python train_v2.py
python importance_v2.py

Key network feature importances identified include cqi (0.394), snr (0.274), dl_mcs (0.163), and rsrp (0.120) [cite: 1].

## 3. Python Synthetic Augmentation & Smart Offloading Dataset Construction

To model computational offloading decisions accurately, the real srsRAN network metrics were augmented using Python [cite: 1, 2]. Synthetic device parameters, application requirements, mobility dynamics, and multi-tier server distances were integrated into the real network baseline [cite: 1, 2].

### 3.1 Feature Definitions

- Real Network Features: CQI, SNR, RSRP, Downlink/Uplink Error counts [cite: 1, 2].

- Synthetic Device Features: Battery Level (%), CPU Load (%), RAM Usage (%), Device Temperature (°C) [cite: 1, 2].

- Application Features: Task Size (MB), Task Complexity (1–10 scale), Latency Requirement (ms), Bandwidth Requirement (Mbps) [cite: 1, 2].

- Contextual & Infrastructure Features: Mobility Speed (km/h), Edge Distance (km), Cloud Distance (km) [cite: 1, 2].

### 3.2 Network Quality Scoring Function

Network quality is categorized into GOOD (2), MEDIUM (1), and POOR (0) based on the following heuristic evaluation rule [cite: 2]:

- +2 points if CQI ≥ 12; +1 point if 8 ≤ CQI < 12 [cite: 2]

- +2 points if SNR ≥ 20 dB; +1 point if 10 ≤ SNR < 20 dB [cite: 2]

- +2 points if RSRP > -90 dBm; +1 point if -105 < RSRP ≤ -90 dBm [cite: 2]

- +1 point if DL Error == 0; +1 point if UL Error == 0 [cite: 2]

Quality Classification: Score ≥ 6 → GOOD (2); Score 3–5 → MEDIUM (1); Score < 3 → POOR (0) [cite: 2].

### 3.3 Cost-Based Decision Engine and Label Generation

The optimal offloading location (Mobile = 0, Edge = 1, Cloud = 2) is selected by calculating the computational and communication costs for each execution tier [cite: 1, 2]:

# Cost Formulation
Mobile Cost = (Task_Size * 0.60) + (Complexity * 20) + (CPU * 0.40) + (RAM * 0.10) + ((100 - Battery) * 0.70) + N(0, 15)

Edge Cost   = (Edge_Distance * 4) + (Latency_Req * 0.50) + (Task_Size * 0.15) + (Complexity * 8) - (Network_Quality * 25) - (Bandwidth * 0.20) + N(0, 15)

Cloud Cost  = (Cloud_Distance * 0.50) + (Latency_Req * 1.50) + (Task_Size * 0.05) - (Complexity * 8) - (Bandwidth * 0.10) + N(0, 15)

Controlled Gaussian noise (N(0,15)) is added to each cost function to model real-world system variance and unexpected network jitter [cite: 1, 2]. The final target label is assigned as argmin(Mobile_Cost, Edge_Cost, Cloud_Cost) [cite: 2].

### 3.4 Full Python Pipeline Implementation

Below is the complete Python script used to synthesize parameters, evaluate costs, assign offloading labels, calculate confidence scores, and export the dataset [cite: 2]:

import pandas as pd
import numpy as np

# Configuration
INPUT_FILE = "final_dataset_augmented.csv"
OUTPUT_FILE = "smart_offloading_dataset_v2.csv"
AUGMENT_FACTOR = 5

np.random.seed(42)

# Load base network dataset
df = pd.read_csv(INPUT_FILE)

# Network Quality Score Calculation
def calculate_network_quality(row):
    score = 0
    if row["cqi"] >= 12: score += 2
    elif row["cqi"] >= 8: score += 1

    if row["snr"] >= 20: score += 2
    elif row["snr"] >= 10: score += 1

    if row["rsrp"] > -90: score += 2
    elif row["rsrp"] > -105: score += 1

    if row["dl_error"] == 0: score += 1
    if row["ul_error"] == 0: score += 1

    if score >= 6: return 2      # GOOD
    elif score >= 3: return 1    # MEDIUM
    return 0                    # POOR

# Cost Calculation Engine
def calculate_costs(row):
    battery = row["battery_level"]
    cpu = row["cpu_load"]
    ram = row["ram_usage"]
    task_size = row["task_size_mb"]
    complexity = row["task_complexity"]
    latency = row["latency_requirement_ms"]
    network = row["network_quality"]
    edge_distance = row["edge_distance_km"]
    cloud_distance = row["cloud_distance_km"]
    bandwidth = row["bandwidth_requirement"]

    # Cost Formulas
    mobile_cost = (task_size * 0.60 + complexity * 20 + cpu * 0.40 + ram * 0.10 + (100 - battery) * 0.70)
    edge_cost = (edge_distance * 4 + latency * 0.50 + task_size * 0.15 + complexity * 8 - network * 25 - bandwidth * 0.20)
    cloud_cost = (cloud_distance * 0.50 + latency * 1.50 + task_size * 0.05 - complexity * 8 - bandwidth * 0.10)

    # Realistic random noise addition
    mobile_cost += np.random.normal(0, 15)
    edge_cost += np.random.normal(0, 15)
    cloud_cost += np.random.normal(0, 15)

    return mobile_cost, edge_cost, cloud_cost

# Dataset Generation Loop
rows = []
for _, row in df.iterrows():
    for _ in range(AUGMENT_FACTOR):
        new_row = row.copy()

        # Synthetic Device Features
        new_row["battery_level"] = np.random.randint(5, 101)
        new_row["cpu_load"] = np.random.randint(5, 101)
        new_row["ram_usage"] = np.random.randint(10, 101)
        new_row["device_temperature"] = round(np.random.uniform(30, 55), 2)

        # Synthetic Application Features
        new_row["task_size_mb"] = np.random.randint(1, 2000)
        new_row["task_complexity"] = np.random.randint(1, 11)
        new_row["latency_requirement_ms"] = np.random.randint(5, 200)
        new_row["bandwidth_requirement"] = round(np.random.uniform(1, 100), 2)

        # User Mobility & Infrastructure Features
        new_row["mobility_speed"] = round(np.random.uniform(0, 120), 2)
        new_row["edge_distance_km"] = round(np.random.uniform(1, 30), 2)
        new_row["cloud_distance_km"] = round(np.random.uniform(50, 1000), 2)

        # Derived Metrics
        new_row["network_quality"] = calculate_network_quality(new_row)
        m_cost, e_cost, c_cost = calculate_costs(new_row)

        new_row["mobile_cost"] = round(m_cost, 2)
        new_row["edge_cost"] = round(e_cost, 2)
        new_row["cloud_cost"] = round(c_cost, 2)

        # Target Label & Confidence Score
        costs = [m_cost, e_cost, c_cost]
        new_row["label"] = int(np.argmin(costs))

        sorted_costs = sorted(costs)
        confidence = (sorted_costs[1] - sorted_costs[0]) / abs(sorted_costs[1])
        new_row["confidence"] = round(max(0, min(confidence, 1)), 3)

        rows.append(new_row)

# Save Final Dataset
final_df = pd.DataFrame(rows)
final_df.to_csv(OUTPUT_FILE, index=False)
print("Saved:", OUTPUT_FILE)